People-Counting System — IP Camera + NVR Reference Manual

Velvu ST-VB IP2002DL camera · Velvu EZ-NVR-9516K1 NVR · YOLOv8 + ByteTrack pipeline · Windows setup

# 1. System Overview

Pipeline: IP Camera (via NVR) → RTSP Stream → Frame Capture → Person Detection (YOLOv8) → Multi-Object Tracking (ByteTrack) → Line-Crossing Logic → Count.

The critical design point: you never count on raw detections. A single person appears in hundreds of consecutive frames, so counting detections directly inflates numbers massively. The pipeline must:

- Detect people in each frame (YOLOv8, person class only).

- Track each person with a persistent ID across frames (ByteTrack, built into Ultralytics).

- Increment the count only once per tracked ID, when that ID's centroid crosses a defined virtual line.

# 2. Hardware in Use

## 2.1 Camera — Velvu ST-VB IP2002DL

| Spec | Value |
| --- | --- |
| Type | 2MP IP Bullet Camera |
| Features | Built-in audio, 24/7 full-color night vision, metal body |
| Ingress protection | IP67 (outdoor rated) |
| Lens | 3.6 mm fixed |
| Compliance | ONVIF-compliant |
| Connection | RJ45 (wired Ethernet), typically PoE-powered via NVR |

## 2.2 Recorder — Velvu EZ-NVR-9516K1

A networked video recorder (NVR) that all your Velvu IP cameras plug into over PoE/RJ45. The NVR itself becomes the single network endpoint your Python pipeline talks to — you do not need to connect to each camera's own IP separately once it's registered on the NVR.

The "95" prefix pattern and "16" channel count in this class of Indian-market OEM NVR (commonly Hikvision-derivative firmware, white-labelled) typically indicates a 16-channel PoE NVR. Confirm your exact channel count from the unit's front panel / spec sheet, since RTSP channel numbering depends on it.

Note: Velvu NVRs in this line are commonly built on Hikvision-compatible firmware. This means the RTSP conventions below (Section 4) are the first thing to try — but always confirm via ONVIF or the NVR's own web UI, since OEM firmware forks occasionally diverge.

# 3. Software Stack (Windows)

| Component | Purpose |
| --- | --- |
| Python 3.10+ | Runtime |
| OpenCV (cv2) | RTSP capture, drawing, frame ops |
| Ultralytics YOLOv8 | Detection + built-in ByteTrack tracking |
| VLC Media Player | Verifying the RTSP URL works before writing any code |
| ONVIF Device Manager | Windows GUI tool to discover exact RTSP URIs — avoids guessing |

# 4. Getting the RTSP Stream from the NVR

Since all cameras are wired into the NVR, you pull the stream from the NVR's IP — not the camera's individual IP. The NVR exposes one RTSP port (554) and differentiates cameras by channel number in the URL path.

## 4.1 Step 1 — Find the NVR's IP address

- Check your router's DHCP client list / connected devices for a device named "NVR" or the Velvu model string.

- Or, on Windows, download and run a Hikvision-compatible discovery tool (SADP Tool works with most OEM-derivative NVRs of this type) — it lists the NVR's IP, MAC and serial without needing credentials.

- Or scan your subnet: open PowerShell and run a simple port sweep on 554/80/8000 across your LAN range.

## 4.2 Step 2 — Try the standard NVR RTSP path

For Hikvision-convention NVRs (which this model is very likely built on), the RTSP path encodes channel + stream type as a 3-digit suffix: channel number, then 01 for main stream / 02 for sub-stream.

rtsp://<username>:<password>@<NVR_IP>:554/Streaming/Channels/101   # Channel 1, main stream

rtsp://<username>:<password>@<NVR_IP>:554/Streaming/Channels/102   # Channel 1, sub stream

rtsp://<username>:<password>@<NVR_IP>:554/Streaming/Channels/201   # Channel 2, main stream

rtsp://<username>:<password>@<NVR_IP>:554/Streaming/Channels/1601  # Channel 16, main stream

If that path returns nothing, try the Dahua-style convention as a fallback (some OEM NVR firmware forks use this instead):

rtsp://<username>:<password>@<NVR_IP>:554/cam/realmonitor?channel=1&subtype=0   # Channel 1, main

rtsp://<username>:<password>@<NVR_IP>:554/cam/realmonitor?channel=1&subtype=1   # Channel 1, sub

## 4.3 Step 3 — Verify in VLC before writing any Python

- Open VLC → Media → Open Network Stream.

- Paste the RTSP URL with your actual NVR IP, admin username and password.

- If video plays, that exact string is what goes into your Python script's rtsp_url variable.

- If it fails, open a browser to http://<NVR_IP> and log into the NVR's own web UI — under Network / RTSP settings, most NVRs of this class will display the exact RTSP port and per-channel path directly.

Note: Default NVR credentials are almost always admin / (blank or admin) on first boot, but Velvu — like most Indian-market OEM brands — typically forces a password change via its companion mobile app or web UI on first login. Check the sticker on the NVR chassis if unsure.

## 4.4 Step 4 — Confirm via ONVIF (most reliable, skips all guessing)

Since both the camera and (very likely) the NVR are ONVIF-compliant, you can query the exact stream URI programmatically instead of guessing paths:

pip install onvif-zeep

from onvif import ONVIFCamera

cam = ONVIFCamera('<NVR_IP>', 80, 'admin', '<password>')

media_service = cam.create_media_service()

profiles = media_service.GetProfiles()

for i, profile in enumerate(profiles):

uri = media_service.GetStreamUri({

'StreamSetup': {'Stream': 'RTP-Unicast', 'Transport': {'Protocol': 'RTSP'}},

'ProfileToken': profile.token

})

print(f'Channel/profile {i}: {uri.Uri}')

# 5. Windows Environment Setup

- Install Python 3.10+ from python.org — during install, check "Add Python to PATH".

- Open PowerShell (or your Antigravity integrated terminal) in your project folder.

- Create and activate a virtual environment:

python -m venv venv

venv\Scripts\activate

- Install dependencies:

pip install ultralytics opencv-python onvif-zeep

- YOLOv8 will auto-download the nano weights (yolov8n.pt) on first run — no separate download step needed.

# 6. Full People-Counting Script

This is a complete, working script. Paste it directly into a .py file in Antigravity (or any editor) — just replace the rtsp_url with the value you confirmed in Section 4.

import cv2

from ultralytics import YOLO

from collections import defaultdict

model = YOLO("yolov8n.pt")

# Replace with the RTSP URL confirmed via VLC/ONVIF in Section 4

rtsp_url = "rtsp://admin:<password>@<NVR_IP>:554/Streaming/Channels/101"

cap = cv2.VideoCapture(rtsp_url, cv2.CAP_FFMPEG)

cap.set(cv2.CAP_PROP_BUFFERSIZE, 1)  # reduce latency drift

LINE_Y = 300  # y-coordinate of the virtual counting line, in pixels

track_history = defaultdict(list)   # track_id -> recent centroid y-values

counted_ids = set()

count_in = 0

count_out = 0

while cap.isOpened():

ret, frame = cap.read()

if not ret:

continue

results = model.track(

frame, persist=True, classes=[0], verbose=False, tracker="bytetrack.yaml"

)

if results[0].boxes.id is not None:

boxes = results[0].boxes.xywh.cpu()

track_ids = results[0].boxes.id.int().cpu().tolist()

for box, track_id in zip(boxes, track_ids):

x, y, w, h = box

cy = float(y)

track_history[track_id].append(cy)

if len(track_history[track_id]) > 30:

track_history[track_id].pop(0)

if len(track_history[track_id]) >= 2 and track_id not in counted_ids:

prev_cy = track_history[track_id][-2]

if prev_cy < LINE_Y <= cy:

count_in += 1

counted_ids.add(track_id)

elif prev_cy > LINE_Y >= cy:

count_out += 1

counted_ids.add(track_id)

annotated = results[0].plot()

cv2.line(annotated, (0, LINE_Y), (annotated.shape[1], LINE_Y), (0, 255, 0), 2)

cv2.putText(annotated, f"IN: {count_in}  OUT: {count_out}",

(20, 40), cv2.FONT_HERSHEY_SIMPLEX, 1, (0, 0, 255), 2)

cv2.imshow("People Counter", annotated)

if cv2.waitKey(1) & 0xFF == ord('q'):

break

cap.release()

cv2.destroyAllWindows()

How the counting logic works:

- model.track(persist=True) keeps the same ID for a person across frames as long as ByteTrack can associate detections — this survives brief occlusion.

- counted_ids ensures each track ID increments the count exactly once, even if the person lingers near the line.

- Crossing is detected by comparing the previous centroid position to the current one, relative to LINE_Y — this is what prevents double-counting someone standing near the line.

# 7. Handling Multiple Channels from the Same NVR

Run one process per channel — each needs its own VideoCapture and its own tracker instance (trackers must not be shared across channels, or IDs will collide across cameras).

import multiprocessing

channels = [101, 201, 301]  # main streams for channels 1, 2, 3

def run_counter(channel):

rtsp_url = f"rtsp://admin:<password>@<NVR_IP>:554/Streaming/Channels/{channel}"

# ... same pipeline as Section 6, parameterized by rtsp_url ...

if __name__ == '__main__':

for ch in channels:

multiprocessing.Process(target=run_counter, args=(ch,)).start()

Aggregate counts across processes via a shared SQLite file, Redis, or a small local FastAPI endpoint each process posts to.

# 8. Production Hardening

## 8.1 RTSP reconnect loop

NVR-fed RTSP streams drop occasionally over long runs (network hiccups, NVR reboot). Wrap capture in a reconnect loop:

def robust_capture(rtsp_url):

cap = cv2.VideoCapture(rtsp_url, cv2.CAP_FFMPEG)

while True:

ret, frame = cap.read()

if not ret:

cap.release()

cap = cv2.VideoCapture(rtsp_url, cv2.CAP_FFMPEG)  # reconnect

continue

yield frame

## 8.2 Other considerations

- Frame skipping: process every 2nd–3rd frame on modest hardware — tracking tolerates some frame gaps.

- Line placement: perpendicular to the dominant walking direction; avoid spots where people pause (causes flicker across the line → double counts). Add a small dead-zone band around the line if this happens.

- Crowd occlusion: top-down camera mounting reduces person-to-person occlusion versus angled mounting.

- Model size: yolov8n is enough for single-class (person-only) counting — no need for yolov8x unless compute is free.

# 9. Calibration & Validation

- Manually count people entering over a 15–30 minute window while the system runs.

- Compare against the script's output — expect 90–98% accuracy with good camera placement.

- Common failure mode: undercounting when two people cross shoulder-to-shoulder (bounding boxes merge). Mitigate with camera height that gives more vertical separation between heads.

# 10. Optional Extensions

- Zone-based (not line-based) counting for live occupancy — use a polygon zone instead of a line, tracking entry/exit into the zone.

- Head-only detection for dense crowds where bodies are occluded but heads are visible — requires fine-tuning a head-detection model rather than stock COCO person class.

- Pipe counts into Grafana/InfluxDB for a historical dashboard.

# 11. Quick Troubleshooting Checklist

| Symptom | Likely Cause / Fix |
| --- | --- |
| VLC can't connect to RTSP URL | Wrong credentials, wrong channel number, or RTSP port blocked — check NVR web UI network settings |
| VLC works, but cv2.VideoCapture returns ret=False | Missing FFmpeg backend — ensure opencv-python (not headless-only builds without codecs) and pass cv2.CAP_FFMPEG explicitly |
| Counts far too high | Tracker not persisting IDs correctly — confirm persist=True and that you're not creating a new YOLO() model instance per frame |
| Counts too low / missed people | Line placed where people pause, or heavy occlusion — reposition line or camera angle per Section 8.2 |
| Stream freezes after a few minutes | RTSP drop — implement the reconnect loop in Section 8.1 |